Consultation exercise for a proposed recreational amenity trail project, utilising the disused railway line in North Kerry, from the Limerick / Kerry border to Listowel Town

1. Introduction

1.1 Location and background

This amenity trail proposal will be approx 10Km in length and for the most part will be located along the disused railway line from the John B Keane Road, Listowel Town to the Kerry / Limerick County Boundary at Rathoran, Co Kerry. It is also proposed to link the amenity trail with Listowel Town Centre and the Town Park so as to promote economic and recreational synergies and to make use of existing infrastructure. The amenity trail proposal will connect onto the existing 39km Great Southern Trail which recently completed by Limerick County Council. It is considered that this project has the potential to become a valuable recreational, economic and social resource for the area.

The former Great Southern and Western Railway, Limerick to Tralee line was completed in 1880 and operated until its closure in November 1975. Since then scrub type vegetation has encroached onto the line and in places it is used by adjoining farmers for access and fodder storage purposes. The Eirgrid transmission network also crosses the line and a fiber optic cable runs along its length. It is understood that there is an active gun club in the general area (Listowel & District Gun Club).

Kerry County Council in conjunction with Listowel Town Council is preparing a development consent application for this amenity trail proposal. It is the aim of the Council to prepare the proposal in a sustainable manner which minimises the potential for impact on landowners / users in the area and the environment generally. In order to achieve these aims and to facilitate public involvement and participation, an overview of anticipated works and the planning and environmental context are set out below. This consultation exercise is in addition to statutory requirements, which will require the invitation of submissions and observations with respect to the proposed development at a later date, in accordance with the requirements of the Planning and Development Act 2000, as amended.

Section 2 of this document provides an outline of anticipated works. Following on from this an overview of the Planning and Environmental context are set out in section 3, while an overview of other significant plans and projects in the area are set out in section 4.

As part of this current consultation exercise, general submissions in relation to this project are welcome and may be submitted in writing to: Administration Officer, Roads and Transportation, Kerry County Council, Áras an Chontae, Rathass, Tralee, Co. Kerry or by email to: roads@kerrycoco.ie and must be received by the 19th of February 2014 with ‘North Kerry Amenity Trail’ in the subject line.

2. Proposed works

The proposed works will generally be consistent with those recently carried out on the disused railway line between Abbeyfeale and the Limerick / Kerry Boundary. However, it is intended in this instance and subject to funding, to seal the surface of the trail with a bound macadam surface.

It is anticipated that the proposal will include the following:-

- Overburden removal / scrub clearance, where necessary.

- Surface improvement (Consistent sealed surface rolled Cl804 limestone with a bound macadam surface). Fill where required will consist of locally sourced pencil shale. The width of the trail will be approximately 3m.

- Bridge repair / replacement, as necessary.

- Drainage works (reinstatement of drainage channels etc and incorporation of standard sustainable urban drainage systems and sediment control measures, where necessary)

- Standard best practice water pollution control measures to be employed at construction state

- Signage

3. Planning Context and Environmental designations in the area

3.1 Settlement, Recreational and Tourism Strategies

This proposed development is in part located in the functional areas of Kerry County Council and Listowel Town Council, both of which are located in the South West Planning Region.

Listowel Town is a Regional Town of importance and excluding the joint-hub towns of Tralee and Killarney it is the largest settlement in the County.    There are a number of objectives and policies both in the Kerry County Development Plan 2009-2015 and the Listowel Town Development Plan 2010-2016, which support the use of the railway line for recreational purposes.

Fáilte Ireland South West has outlined priorities and goals for the region’s tourism around four themes, one of which is ‘nature-based activities from walking to adventure’. The national transport policy to 2020, Smarter Travel: A Sustainable Transport Future and Ireland’s first National Cycling Policy Framework were both adopted in 2009.These policies introduced a fundamental change to travel and transport in general in Ireland and seek to promote a culture of walking and cycling not just in urban areas and to provide viable alternatives to car based transport and to minimise interaction with motorised vehicles. It is noted that the amenity trail route which forms part of this proposal, has been identified and is included in the National Cycle Network scoping studies supporting maps.

3.2 Natural Heritage

The disused rail line marks the outer edge of the Lower River Shannon cSAC at a four locations (Dromin Lower, Skehanierin Lower, Bunagarha & Kilmeany and Lacka East). Its southern embankment is in places located within the SAC boundaries, but the line itself is not.

The railway line is also located approx 285m away from and downstream of the nearest edge of the Moanveanlagh Bog SAC and approx 360m away from and downstream of the nearest edge of the Stack's to Mullaghareirk Mountains, West Limerick Hills and Mount Eagle SPA.

Other designated nature conservation sites of importance, which are located in the wider area include:-

- River Shannon and Fergus Estuaries SPA

- Cashen River Estuary pNHA

- Moanveanlagh Bog NHA

Surface water run-off from the railway line drains to the River Feale, which in addition to forming part of the Lower River Shannon cSAC is an important Salmonid Fishery. Accordingly, the need to safeguard water quality downstream will be taken into consideration at design stage.

It is considered that the proposed works, which will largely be located on previously disturbed ground associated with the disused railway line, are not likely to significantly impact on soil resources or on any areas of geological or ecological importance. Nonetheless the potential for impact on these natural heritage interests taken into consideration in more detail at design stage.

3.3 Amenities of properties in the vicinity

For the most part the disused railway line adjoins agricultural grassland and woodlands.    On the eastern outskirts of Listowel Town, the line adjoins the Kenny Heights and Woodview housing estates. Elsewhere, the line adjoins a number of one-off house type development sites, particularly in the Bunagarha and Kilmeany areas and in Kilmorna village where an old railway station building is located. In addition, farmyards adjoin the line and in places it is used for the storage of round silage bales and for field access purposes.

Consideration will be given to the protection of amenities of properties in the vicinity at design stage and to accommodate additional compatible uses, as appropriate.

3.4 Landscape

There are a number of listed protected views in the Feale Valley between Listowel Town and the Kerry / Limerick county boundary. These are illustrated in the Kerry County Development Plan 2009-2015. In addition, an extensive area west of Listowel Town is zoned as secondary special amenity value in the plan.

While this aspect will be reviewed in more detail at design stage, it is considered that amenity trail type development proposals which seek to utilise disused railway lines, do not generally by their nature, result in substantial landscape recourse change or significantly impact on protected views or prospects.

3.5 Cultural Heritage

Listowel Town is a designated Heritage Town and its railway infrastructure forms part of that heritage. There are a number of Protected Structures and Architectural Conservation Areas located in Listowel Town the closest being the Red Cottages on the Listowel to Tarbert Road. There are also a number of places of archaeological interest located in the area. These sensitivities will be taken into account at design stage.

3.6 Flood Risk Management

The proposal is not considered to be a flood vulnerable one and will be designed having regard to the DEHLG (2009) guidelines on the Planning System and Flood Risk Management.

4. Potential for cumulative / in combination effects

When formulating a proposal consideration is given to the potential for cumulative and in combination effects with other plans and projects in the area and would welcome your views in this regard. Other plans and projects in the area include the following:-

4.1 The Listowel Bypass.

A route has been selected for the N69 Listowel bypass and Environmental Assessments for the scheme are currently being prepared. It is noted that the route selected is located to the west of the town and seeks to use part of the Listowel to Tralee rail line.

4.2 The Shannon Catchment Flood Risk Management Plan

This plan aims in part to prepare a Flood Risk Management Plan for the area that sets out how OPW, local authorities and stakeholders should work together to create sustainable and cost-effective ways of dealing with flood risk now and into the future. As part of this process Listowel Town has been identified as an Area of Further Assessment (AFA).

4.3 The Shannon River Basin Management Plan

This plan sets out measures to be taken by stakeholders and the tools to prioritise the application of available resources to those measures, with the aim of achieving the objectives of the Water Framework Directive. As part of this, member states must generally ensure that their waters achieve at least good status and that status doesn’t deteriorate in any waters.

4.4 Other recreational infrastructure projects

It is proposed that this amenity trail will link in with the existing southern amenity trail (Kerry / Limerick Boundary to Rathkeale). It is also recognised that this may extend as far as Tralee and Fenit in the future. There is also the potential to link in with existing recreational walks in the area including those in the town park and along the riverside.